Auto Probe PCB

# 1.  Summary

This program converts a probe card’s pinout CSV into a fully functioning component in Altium. Running the script produces three scripts written in Altium’s DelphiScript language, plus a wiring map. In Altium these scripts create a schematic component for each net class in the CSV, and a PCB footprint that must be linked to its respective schematic.

# 2.  Running the generator

Auto Probe PCB/
├── Scripts/auto_probe_pcb.py       the generator
├── auto_probe_pcb_inputs/          put exactly ONE .csv here
└── auto_probe_pcb_outputs/         generated files land here

From Scripts/, run:

py auto_probe_pcb.py

It picks up the single CSV in the inputs folder and writes four files to the outputs folder. If there are zero or multiple CSVs it stops with an error rather than guessing. Paths are relative to the script, so the whole folder can be moved without editing anything.

The console reports the per-edge pad counts, the land-ring size, keep-out clearances and the minimum land-to-card-edge clearance. If the fan-out ever overruns the card, that last figure turns into a warning.

# 3.  Input CSV

The header row must contain X (um) and Y (um). Column names are case-insensitive and extra columns are ignored. Rows without parsable coordinates are skipped silently, so blank separator rows and annotation lines in a real pinout are harmless. Pads with no Net Class fall into a group named UNCLASSIFIED.

| Column | Required | Purpose |
| --- | --- | --- |
| X (um), Y (um) | yes | die bond-pad coordinates |
| Pad | no | chip pad name, carried into the wiring map |
| Signal | no | net name — becomes the silk label and schematic pin name |
| Net Class | no | groups lands into schematic components (see §5) |

# 4.  Altium workflow

| File | Run it in | Builds |
| --- | --- | --- |
| <stem>_probe_card.pas | PcbDoc | the complete board |
| <stem>_probecard_footprint.pas | PcbLib | footprints, one per net class |
| <stem>_probecard_symbol.pas | SchLib | components, one per net class |
| <stem>_wiring_map.csv | — | cross-reference: land id, edge, row, signal, die pad, coordinates |

1.  Open a PcbLib, make it the active tab, run GenerateFootprint in File > Run Script.

2.  Open a SchLib, make it the active tab, run GenerateSymbol.

3.  On each schematic component, in the editor window, go to Edit > Browse and link the corresponding footprint for each of the schematic components. After doing so the footprint can be seen when highlighting each of the components.

4.  In the SchLib, the Place button can be clicked to have the highlighted component be placed in an existing SchDoc.

5.  Create a PcbDoc then go to Project > Compile PCB Project, then Design > Update PCB Document > Validate Changes > Execute Changes.

6.  After the parts have been generated on the PcbDoc, each subset of probe lands will be seen, and must be aligned manually to the same X/Y.

*Save the entire project between each step

# 5.  The five-component split

Why five components and not one five-part component. Altium allows exactly one footprint per component, so a single multi-part symbol can only carry one footprint. Splitting into five single-part components lets each own its matching footprint.

Differential pairs are merged. Net classes differing only by a trailing ‘ P’/‘ N’ are combined into one component, with each P land placed directly above its N partner. Partners are matched by signal names differing in exactly one character position, so inconsistent naming still pairs correctly.

How lands are divided. Two independent splits happen. The physical one classifies each pad T/B/L/R and numbers it along its edge, that becomes the pin designator and the pad name. The logical one buckets the same lands by Net Class into components. Because the two are independent, one component’s pins are normally scattered around the die.

Pin designators equal pad names (T25, L16, B4), so Altium maps pins to pads automatically with no explicit pin map. Every land appears exactly once, in exactly one component.

# 6.  Geometry and layout

Land ring. Lands sit on a rectangle that is the keep-out scaled up. The required scale takes the larger of the keep-out clearance floor and each edge’s row-length demand, so the ring follows the keep-out’s aspect ratio rather than the die’s.

Edge assignment. Each pad is classified T/B/L/R by comparing |nx| vs |ny| normalised to the die.

Two staggered rows. Lands alternate between an inner and outer row, halving the along-edge step.

Aperture. A circular board cutout of radius die half-diagonal + APERTURE_CLEARANCE, built as a board-cutout region.

## Layers

| Layer | Contents | Fabricated |
| --- | --- | --- |
| Multi-Layer | the plated through-hole lands | yes |
| Top Overlay | signal labels | yes (ink) |
| board outline / cutout | perimeter and aperture | yes |
| Mechanical 13 | probe needles (land → die pad) | no |
| Mechanical 15 | keep-out rectangle | no |

# 7.  Tunable parameters

Every value below lives in one block at the top of auto_probe_pcb.py. All are µm unless noted. Nothing else in the script needs editing to re-target a different die or card.

## Die and lands

| Constant | Meaning |
| --- | --- |
| DIE_X, DIE_Y | die size |
| PROBE_PAD_SIDE | land outer diameter |
| VIA_DRILL | plated hole, 0.020″, Accuprobe’s recommended floor |
| PROBE_PAD_CLEARANCE | copper gap between adjacent lands |
| PROBE_WIRE_WIDTH | drawn width of the reference needle tracks |

## Rows and stagger

| Constant | Meaning |
| --- | --- |
| LAND_ROWS | rows of lands per edge; 1 disables the stagger |
| NEEDLE_CLEARANCE | needle-to-land margin used to size the row pitch (0.005″) |

## Board, aperture and keep-out

| Constant | Meaning |
| --- | --- |
| BOARD_WIDTH | card width (114.5 mm) |
| BOARD_HEIGHT | card height (188.5 mm) |
| BOARD_CENTER | board centre in Altium sheet coordinates |
| APERTURE_CLEARANCE | die corner → aperture edge |
| KEEP_OUT_WIDTH | keep-out rectangle width |
| KEEP_OUT_HEIGHT | keep-out rectangle height |
| LAND_KEEPOUT_GAP | minimum land-to-keep-out clearance |

## Silkscreen and markers

| Constant | Meaning |
| --- | --- |
| LABEL_SIZE | silk text height |
| LABEL_THICK | silk stroke width |
| LABEL_GAP | gap from land edge to the start of its label |
| MARKER_LINE | keep-out marker line width |

## Schematic symbol — mils, not µm

| Constant | Meaning |
| --- | --- |
| SCH_PIN_PITCH | vertical spacing between pins |
| SCH_PIN_LENGTH | pin stub length |
| SCH_BLOCK_WIDTH | component body width |

## Derived

These are computed from the values above. Editing them in place breaks the relationship they encode; change their inputs instead.

| Constant | Formula |
| --- | --- |
| PROBE_PAD_PITCH | PROBE_PAD_SIDE + PROBE_PAD_CLEARANCE |
| ROW_GAP | PROBE_PAD_PITCH |
| ROW_PITCH | max(PROBE_PAD_PITCH, PROBE_PAD_SIDE + PROBE_WIRE_WIDTH + 2×NEEDLE_CLEARANCE) |
| STAGGER_STEP | ROW_PITCH / LAND_ROWS |
| annular ring | (PROBE_PAD_SIDE − VIA_DRILL) / 2 |

# 8.  Accuprobe requirements

From correspondence with the vendor:

Probe needles are Ø 0.010″. For the smallest design, vias of Ø 0.020″ work; going smaller is possible but increases both fabrication and assembly difficulty at Accuprobe.

SMD pads with vias are ideal, but if only one, prefer vias.

Install components before attaching the ring assembly, to avoid damaging the probe pins. Components therefore belong on the top side, clear of the landing region.

Board-to-wafer standoff is Accuprobe’s domain, set by needle geometry and the epoxy ring.

Current design is via-only (a plated through-hole land is the via), which is Accuprobe’s recommended fallback and the smallest option.

# 9.  Duplicate components in the schematic library

Running GenerateSymbol occasionally produces extra copies unintentionally.

Cause. Altium’s Lib.AddSchComponent always adds and never replaces a same-named component, so the count grows by one on every run and is saved into the .SchLib. Deleting the entry in the SCH Library panel does not always purge it from the file.

Fix. Close Altium, delete the cached folders under %APPDATA%/Altium, empty the recycle bin, and reopen the project.