Probe-Card Contact-Test Mask Generator

Design, Implementation and Reference

# What the project does

This tool builds a test chip that identifies which pads are being probed from a single resistance measurement. Its primary use case is groups of signals whose inputs are shorted together externally on the probe card. Such a group cannot be debugged with a two-point measurement, because the short collapses every probe into a single node. The chip instead assigns each landed probe a unique resistance value, so a simple decoding calculation reveals which probes are not making contact.

Beyond this primary function, the tool can also generate a simple coupon with all probe pads shorted together, intended for testing inputs that are not shorted externally. Every output is additionally accompanied by a two-point-measurable calibration coupon containing a copy of each resistor value used in any of the parallel layouts on the other chips.

## The measurement

Each input pad is wired through a resistor of a known, unique value to a shared node, visible as a rectangular ring inside the pad ring. This ring also connects to the designated output pads, completing the parallel resistance measurement. Because the input pads are shorted together on the probe card, measuring from that node to the output reads every landed resistor in parallel. A probe that fails to land drops its resistor out of the network, raising the reading.

Resistor values follow a binary ladder, which is the optimal arrangement for maximizing noise headroom in the decoding step. The key statistic to watch when changing settings is the decode margin, which the program prints on every run. Probes add in CONDUCTANCE, so the margin is measured there: it is the smallest gap between any two of the possible readings, expressed as a fraction of the all-landed total. On a binary ladder that smallest gap is the one between all resistors present and the largest resistor missing, because the largest resistor contributes the least conductance. Any measurement error must stay well inside half that gap for decoding to remain unambiguous. Note the margin is a conductance fraction, not a resistance ratio: the same pair expressed as a percentage change in measured ohms looks slightly larger, so compare against the printed figure rather than recomputing from resistances.

| Resistors per reading | Best possible margin |
| --- | --- |
| 4 | 6.67 % |
| 5 | 3.23 % |
| 6 | 1.59 % |
| 8 | 0.39 % |

## The three chip types

Parallel coupon - the normal case. Inputs are shorted externally, so they are read together as one parallel value and decoded.

Continuity coupon - for inputs that are not shorted externally. All pads are shorted on-chip with no resistors, and each probe is tested on its own: landed reads a short, open reads infinite.

Calibration coupon – used before taking real measurements to calibrate the decoder.

# Input pinout format

Column names are case-insensitive; leading blank rows and extra columns are ignored, so an existing probe-card spreadsheet usually works with only the I/O column added. The header row is found by looking for the X and Y columns.

| Column | Required | Meaning |
| --- | --- | --- |
| Pad | yes | Pad name, used in every report and label |
| Signal | no | Net name; shown on schematics for context |
| X (um) | YES | Pad centre X in microns; also locates the header row |
| Y (um) | YES | Pad centre Y in microns |
| I/O | YES | The grouping tag; see the table below |

## The I/O tags

| Tag | Meaning |
| --- | --- |
| INPUTn | An input in group n. Gets a resistor to its reading's node. An input takes exactly ONE group: in two it would need a different ladder rung in each and would perturb both readings. |
| OUTPUTn | An output for group n. Ties straight to that group's node. May name SEVERAL groups, comma separated -- 'OUTPUT1, OUTPUT2' or the shorthand 'OUTPUT1,2' -- and is then wired into each of those groups' coupons. |
| OUTPUTALL | A common output added to EVERY group. Because every coupon claims it, no two coupons can share a chip, so a pinout using OUTPUTALL always builds single-layer chips. |
| INPUTSINGLE | An input on the continuity coupon, for signals that are not shorted externally. |
| OUTPUTSINGLE | The shared return on the continuity coupon. If there are none, the coupon returns through the OUTPUTALL pads, or failing that every group output. |

## Running the generator

When the script is run, the user will be prompted on how many metal layers each chip should have. One layer puts a single IO group on each chip and needs no vias. Two layers puts a second group on metal 2 stitched down to the shared pads with vias.

Two coupons may share a chip only if their pad sets are disjoint. This is because pads claimed by both layers would tie the two groups together and destroy both decodes. Coupons that cannot be paired are given their own single layer chip so an unsafe build is never forced.

## How a coupon is cut to reduce chip count

A user's CSV may contain groups that exceed MAX_BINARY_INPUTS, a limit imposed to preserve adequate decode margin. When this occurs, the program splits the group into the fewest sub-groups that each fall under the limit.

Sub-groups share a single coupon layer and are electrically isolated from one another by cutting openings in the central hollow ring. Because of this isolation, the output pads must also be divided evenly among the sub-groups, with each sub-group retaining at least 2 outputs to preserve a baseline of redundancy. If fewer than 2 outputs per sub-group are available, or if no cut placement divides the inputs evenly while leaving each sub-group with 2 outputs, the program halts with an error rather than silently building the coupon whole.

Continuity and calibration coupons are never split.

Before trusting any split coupon, check how the probe card wires its return probes. The readings on a chip are independent only while their OUTPUT pads are kept apart off-chip. If the card ties all return probes to one common bus, the readings rejoin outside the chip and the isolation cut into the ring counts for nothing. This failure is silent: the layout still looks right and the program still reports a healthy margin. It is also worse than a merely tight reading, because every reading restarts the same ladder, so rejoined readings carry identical resistor values and different contact patterns become genuinely indistinguishable rather than just close together. Each reading must be measured between its OWN output pads and the input rail, so confirm the card can isolate them before committing to a split design.

## Outputs

| Path | Contents |
| --- | --- |
| io_pair_outputs/<pinout>_parallel.csv | The decode key: one row per resistor |
| io_pair_outputs/gds/groups_<n>.gds | One GDS per chip, named for its group or groups |
| io_pair_outputs/schematics/*.svg | A circuit diagram per chip, one block per layer |
| io_pair_outputs/calibration/* | Calibration coupon GDS and its expected values |
| io_pair_outputs/all_tiled_chips.gds | Every die tiled and stepped across the wafer |

## Columns in the decode key

| Column | Meaning |
| --- | --- |
| chip | Which chip and GDS this resistor is on |
| layer | Metal layer, 1 or 2 |
| half | Which reading on that layer, numbered from 1, or 0 when the coupon is not split. Each reading is its own decode unit. A coupon may have more than two. |
| input_pad | The pad, i.e. the probe being tested |
| input_signal | Its net name |
| output_pads | The pads to measure against |
| actual_R_ohm | The resistor's real extracted resistance. Use THIS, not the nominal ladder value. |
| total_len_um | Physical trace length |
| group_parallel_R_ohm | Reading with every probe in that unit landed, or the word continuity |

# Tunable parameters

| Parameter | What it controls |
| --- | --- |
| PAD_SIZE | Probe pad side length in microns. |
| WIRE_WIDTH | Resistor trace width in microns. |
| WIRE_SPACE | Minimum gap between adjacent metal traces in microns, set by the photoresist. |
| COIL_GAP | Gap from the pad edge out to the first comb feature. |
| VIA_SIZE | Via square side, two-layer builds only. |
| PLANE_RING_MARGIN | How far the plane is inset inside the pad ring. |
| FINGER_OVERLAP | How far each output finger and return reaches into the plane. |
| PLANE_SPLIT_GAP | Isolation gap left at each ring cut between neighbouring readings. |
| LEAD_SPACE | Gap for the lead and return wires only -- the run from a pad to the start of its coil and back from the coil to the node. Sets how far those sit from each other and from the coil edge. Where a return threads between two pads it reverts to WIRE_SPACE. |
| CORNER_RADIUS | Fillet radius on every trace corner; 0 draws square corners. Inside corners are rounded as well as outside, so the metal removed at one is given back at the other and the resistance does not move. |
| CORNER_TOLERANCE | Chord tolerance of the fillet arc approximation. Smaller is smoother and larger on disk. |
| PLANE_RING_WIDTH | Wall width of the hollow shared-node ring. Must be at least FINGER_OVERLAP or returns overshoot the wall into the hollow. |
| MAX_BINARY_INPUTS | Most resistors allowed to share ONE reading, and the single biggest lever on the decode margin, which is 1/(2^n - 1) for n resistors. A group with more inputs than this is cut into the fewest readings that each fall under it. |
| DIE_MARGIN_BUFFER | Clearance beyond the farthest resistor before the die edge. |
| METAL_THICKNESS_UM | Deposited metal thickness. Only enters through SHEET_RES. |
| AL_RESISTIVITY | Ohm-metres. Bulk aluminium is 2.65e-8; thin films run 3.0 to 3.5, so this is a sound 100 nm baseline. Do not use the bulk value. |
| SHEET_RES | AL_RESISTIVITY divided by thickness, about 0.324 ohm per square. Sets trace length per ohm. |
| COIL_BASE_R | The smallest resistor in a ladder; each rung doubles. Raising it helps against series contact resistance but multiplies every trace length and so the die area. |
| CORNER_SQUARES | Squares a right-angle bend really conducts. Current crowds the inside of a turn, so a centre-line count over-estimates. About 1 percent residual. |
| CALIB_COUNT | How many ladder rungs get a calibration resistor, and so how many OUTPUT pads carry one. Every remaining output pad becomes a COMMON tap. |
| WAFER_DIAM_UM | Wafer diameter; 150000 is a 6 inch wafer. |
| WAFER_EDGE_EXCLUSION_UM | Unusable edge ring. Fields are packed inside this, though the ring itself is not drawn. |
| MOSAIC_GAP | Spacing between dies inside one tiled field. |
| WAFER_STREET_UM | Gap between stepped fields on the wafer. |
| WAFER_LINE_UM | Width of the drawn wafer-edge outline ring. |
| ADD_LABELS | Draw pad names, IO tags and coupon labels. |
| LABEL_SIZE | Pad text height, microns. |
| ADD_DIE_OUTLINE | Draw the die boundary on layer 100. |
| ROUTE_PITCH | WIRE_WIDTH + WIRE_SPACE. Centre-to-centre pitch of folded wire, so wire area is length times this. |
| CORNER_INSET | PAD_SIZE + WIRE_SPACE. Keeps combs clear of the ring corners. |
| LEAD_RADIUS | Radius of the pad-to-comb lead lane. |
| CANOPY_CLEAR | 3 x ROUTE_PITCH. Radial gap between stacked storeys. |
| CHANNEL_MARGIN | 2 x ROUTE_PITCH. Clearance between a routing channel and the first pad. |
| GDS_MAX_POINTS | Vertices per polygon in the written GDS, the GDSII format ceiling. The library default of 199 fractures long filleted coils and can shatter an arc-dense outline into disconnected slivers. |

## GDS layer map

| Layer | Datatype | Contents | Fabricated |
| --- | --- | --- | --- |
| 1 | 0 | Metal 1: combs, planes, all pads, fingers | YES |
| 2 | 0 | Metal 2, two-layer builds only | yes, if 2-layer |
| 20 | 0 | Via cuts, two-layer builds only | yes, if 2-layer |
| 100 | 0 | Die outline | no, reference |
| 101 | 0 / 1 | Pad names, IO tags, coupon labels | optional |
| 104 | 0 | Wafer edge ring, tiled file only | no, reference |

## How the resistance values are calculated

AL_RESISTIVITY is fixed at 3.243e-8 ohm-metres, an estimate of aluminum deposited on a 10 angstrom titanium layer, and the script uses it to turn each target resistance into a trace length.

Two deviations follow from this, and they are not equivalent. Built resistors land under half a percent below target, though this is harmless because actual_R_ohm is measured back off the finished geometry, so the CSV already describes what was drawn, and the decode compares against it.

A wrong resistivity is a different matter, but it is solved by the calibration coupon that measures the real scale after fabrication and removes it. The constant can also simply be updated once the resistivity has been measured in practice.

## Calibration

Fabricated resistors never match calculated values exactly, because sheet resistance depends on the real deposited film rather than the AL_RESISTIVITY and METAL_THICKNESS_UM you set. Calibrate before trusting any decode. This is a prerequisite.

Land the probe card on the calibration chip. It has the same pad ring as a test chip, so it drops straight on.

One set of output pads carries the resistors and every other output pad is a COMMON tap. They are all the same node, so if a probe misses one, read from another.

Measure each listed pad against any COMMON. The CSV gives the pairs in its measure_from and measure_to columns, and each value is also written next to its pad on the layout.

Add a measured_R_ohm column to the calibration CSV, fill it in, and copy it into decode_inputs. The decoder then scales every prediction by the nearest rung automatically.

# Using the decoder

Copy the decode key from io_pair_outputs into the decode_inputs folder. If you are calibrating, copy the calibration CSV alongside it; the two are told apart by their columns.

Land the probe card. Measure the unit's parallel resistance from the shorted input rail to its output pads. Note the chip, the layer, and on a split coupon which reading it is.

Run py Scripts/find_missing_probes.py, answer the prompts and enter the reading. Values like 470, 1.2k, 3.3M or OPEN are all accepted.

The decoder scores every possible missing set by how well its removal explains the reading, reports the best fit with the decode margin, and warns if another set fits about as well.